# Stage 2 — Interpretation (→ ground model)

Detailed spec for the interpretation stage. This is the first fully-worked stage and the template for the rest: a universal step skeleton, with every code-specific reference held in the Code Pack (see ../02-architecture-and-principles.md P7, decision D21).

Reference legend: ✓ = verified in the source text in ../../Sources/ · ◇ = proposed / standard practice, to confirm.

## Backbone (per pack)

Eurocode 7: EN 1997-2 §1.6 + Figure 1.1 — test results → derived values → (then via EN 1997-1 §2.4.5.2) → characteristic values → design values; plus §6.3 (evaluation) and §6.4 (establishment of derived values). ✓

Australian: AS 1726-2017 §5 — investigation is an iterative process (§5.1.1 ✓), producing a geotechnical model (§5.2 ✓) and reports (§5.6: factual geotechnical data report §5.6.2 ✓; §5.6.2(d) requires regional & local geology ✓).

Universal (core, every pack): the 2.1→2.7 step skeleton, the raw data model, and the rule that the engineer picks the correlation and owns the design value. Only the references/systems below re-point.

## Multi-location (per-location vs site-wide)

A real investigation has many locations (boreholes, CPTs, test pits, lab samples) at different positions and depths. The stage is split along that axis:

Per location (loop): steps 2.1–2.3 run independently at every sounding — correct, classify, derive at each BH/CPT/test.

Site-wide (spatial): steps 2.4–2.7 combine all locations:

2.4 consistency — explicitly cross-location: compare BH vs CPT in the same area, detect outlier/anomalous locations and lateral variability, and compare against prior/nearby/regional/geology data.

2.5 ground model — group soundings into geotechnical units, build cross-sections / fence diagrams (2D/3D), establish the groundwater surface across the site.

2.6 characteristic value — per unit, with a spatial subtlety: EC7 §2.4.5.2's cautious estimate must consider whether the limit state is governed by a local value (pile tip, weak lens) or a spatially-averaged volume (large raft averaging variability) — which depends on the structure. Local-governing vs volume-averaging change the statistic.

2.7 design recommendations — produced per unit/zone.

## Steps and references (side-by-side)

| Step | Eurocode 7 pack | Australian pack |
| --- | --- | --- |
| 2.1 Correct measured values — raw → corrected/normalized per test | EN ISO 22476 methods; SPT energy/overburden Skempton 1986 (Annex F); CPT qₜ Lunne, Robertson & Powell 1997 (Annex D); DMT indices Marchetti 1980 (Annex J) ✓ | AS 1726 §5.5 + App A fieldwork ✓; soil test methods AS 1289 series ◇ |
| 2.2 Classify & behaviour-type | EN ISO 14688-1/-2 (soil), EN ISO 14689 (rock) ✓; CPT soil-behaviour-type via Lunne et al 1997 | AS 1726 §6.1 (soil) / §6.2 (rock) — behavioural, USCS-derived with its own coarse/fine boundary; particle sizes Table 1 ✓ |
| 2.3 Derive parameters (measured → derived) | sanctioned correlations in Annexes D–S — see drill-down below ✓ | AS 1726 §6 = description/classification; derivation correlations engineer-selected (same science), design use governed by AS 2159 / AS 5100.3 ◇ |
| 2.4 Consistency & regional reconciliation | EN 1997-2 §6.3 (evaluation of geotechnical information) ✓ | AS 1726 §5.1.1 iterative ✓; §5.6.2(d) regional & local geology required ✓ |
| 2.5 Stratigraphy / ground model | EN 1997-2 §6; EN 1997-1 §3 ✓ | AS 1726 §5.2 geotechnical model ✓; factual data report §5.6 ✓ |
| 2.6 Value basis (human judgment gate) | characteristic value = cautious estimate — EN 1997-1 §2.4.5.2 ✓; Schneider 1997 ◇; national annex | no single clause; cautiously-assessed strength carried to design via φg (ultimate → design geotechnical strength): AS 2159 §1.3.7 risk-based ✓; AS 5100.3 §7.3.5 ✓ |
| 2.7 Preliminary design recommendations (fₛ/q_b/bearing profiles) | direct field-test design: EN 1997-2 §4.3.4 / §4.4.4 / §4.6.4 / §4.10.4 / §4.11 ✓; piles NEN 6743, Fascicule 62; settlement Schmertmann 1970, Burland & Burbidge 1985 | piles fₛ/q_b & φg: AS 2159 ✓; footing bearing & φg: AS 5100.3 ✓; investigation → params AS 1726 App A/B ◇ |

## Step 2.3 — parameter derivation: scenario-indexed Method Library

Derivation is not one formula per parameter — it is a function of { parameter × soil type × drainage × data available }. At each depth/location the AI reads the actual inputs, filters the library to the applicable methods, proposes a default + alternatives (with scatter), and flags reliability. Weak scenarios (e.g. clay, no lab) still return methods, with a low-confidence flag; the library also encodes when a method is not recommended (e.g. clay φ′ from SPT). Australian pack = same science, engineer-selected; design use per AS 2159 / AS 5100.3.

### Eurocode v1 set (drill-down)

| Parameter | From | v1 method | Eurocode reference |
| --- | --- | --- | --- |
| Undrained strength sᵤ (clay/silt) | Lab | UU/CU triaxial, UCS, fall-cone | CEN ISO/TS 17892-8/-9 ✓; Annex P ✓ |
|  | CPTu | sᵤ = (qₜ − σᵥ₀)/N_kt | Lunne et al 1997 ✓ (Annex D) |
|  | Field vane | sᵤ,fv × μ correction | Aas, Lacasse, Lunne & Høeg 1986 ✓ (Annex I); Bjerrum μ ◇ |
|  | SPT, no lab | sᵤ ≈ f₁·N₆₀ (PI-dep.); sᵤ = 29·N^0.72 | Stroud 1974 ◇; Terzaghi-Peck ◇; Hara et al 1974 ◇ — low reliability |
|  | any (normalized) | sᵤ/σ′ᵥ = S·OCR^m (SHANSEP) | Ladd & Foott 1974 ◇ |
| Effective φ′, c′ — sand | SPT | N → φ′ | EN 1997-2 Annex F ✓; Peck-Hanson-Thornburn / Hatanaka-Uchida 1996 ◇ |
|  | CPT | qc → φ′ | Lunne et al 1997 ✓ (Annex D); Kulhawy & Mayne 1990 ◇ |
|  | Lab | CD/CU triaxial, direct shear | CEN ISO/TS 17892-9/-10 ✓; Annex P ✓ |
| Effective φ′, c′ — clay | Lab | CU(+u)/CD triaxial | CEN ISO/TS 17892-9 ✓ |
|  | no lab | φ′ from plasticity index (φ′↓ with PI) | Mitchell / Mayne ◇ — low reliability; SPT not a reliable route for clay φ′ |
| Relative density Dᵣ | SPT / CPT | (N₁)₆₀ → Dᵣ; qc → Dᵣ | Skempton 1986 ✓ (Annex F); Lunne et al 1997 ✓ (Annex D) |
| Stiffness E / Eₒₑ𝒹 / M | CPT / DMT / PMT | qc → M; Eᴅ → M; Ménard E_M | Schmertmann 1970 ✓; Marchetti 1980/2001 ✓ (Annex J); Clarke 1995 ✓ (Annex E) |
|  | Lab oedometer | Eₒₑ𝒹, mᵥ | CEN ISO/TS 17892-5 ✓; Annex Q ✓ |
| Consolidation Cc, Cr, cᵥ, σ′ₚ, OCR | oedometer | Casagrande / Janbu | CEN ISO/TS 17892-5 ✓; Annex Q ✓; OCR from CPT/DMT ◇ |
| Unit weight γ, index (w, e, PSD, Atterberg) | lab | classification suite | CEN ISO/TS 17892-1/-12 ✓; Annex M ✓ |
| Permeability k | lab / CPTu / grain size | falling-head; dissipation; Hazen | Annex S ✓; CEN ISO/TS 17892-11 ✓; Hazen ◇ |

## Human gates inside interpretation

The engineer (a) picks the correlation per parameter in 2.3, (b) resolves the conflicts the consistency engine raises in 2.4, and (c) owns the value basis in 2.6. Everything else the AI proposes.

## Three divergences that validate the Code Pack split

1. Classification is not shared — AS 1726 deliberately differs from USCS at the coarse/fine boundary; the classification engine is pack-specific.

2. Value basis is the deepest split — EC7 picks a characteristic value then applies partial factors (DA1/2/3); AU assesses strength then applies a risk-based φg (AS 2159). Steps 2.6–2.7 are fully pack-driven.

3. Prescriptiveness differs — EN 1997-2 tabulates sanctioned correlations in annexes; AS 1726 leaves derivation to the engineer, so the Australian pack's 2.3 default is lighter and the Method Library does more of the work.

## Open items (◇ — to confirm with founder)

(a) AS test-method standards for 2.1 (AS 1289 series for soil; CPT/field per AS 1726 App A) — confirm exact references.

(b) Confirm AS 2159 / AS 5100.3 as the anchor for Australian derivation → design parameters.

(c) Step 2.4 reconciliation methodology — founder's preferred statistical/Bayesian references (least code-defined step).

Eurocode ◇ items in the drill-down (Bjerrum μ, Schneider 1997, Kulhawy & Mayne 1990, Hazen) — confirm or swap for house standards.

## Provenance

AS references verified by reading the scanned AS 1726-2017 PDF pages directly (contents p3; §6.1 classification p16; §5.2/§5.6 framing p7/p14 — printed page numbers). AS 2159 / AS 5100.3 verified in their .txt conversions. Eurocode references verified in en.1997.2 / en.1997.1 .txt.

## Task breakdown (operational steps)

🔒 = step gate. Exception gates also fire mid-task on any trigger (see ../03-workflow-and-human-ai-model.md → Gate model). ←CP = supplied by the selected Code Pack.

2.0 — Stage entry & data scoping (once)

2.0.1 Detect entry point (fresh data vs jump-in with existing parameters/ground model — input contract).

2.0.2 List data needed; check presence; request/ingest missing (any format); extract & map fields.

2.0.3 Register the investigation: locations, coordinates, ground levels, test types, dates.

🔒 Gate 2.0 — confirm dataset & scope.

### Per-location loop (every borehole / CPT / test)

2.1 — Correct & normalize

2.1.1 Identify test types & depths.

2.1.2 Validate raw data (units, continuity, ranges, gaps, spikes); flag suspect readings. ←CP

2.1.3 Apply corrections/normalizations (SPT N₆₀/(N₁)₆₀; CPT qₜ, Fr, normalization; DMT indices; vane peak/remoulded). ←CP/library

2.1.4 Reconcile depth datum & water level.

2.1.5 Present corrected profiles.

🔒 Gate 2.1.

2.2 — Classify & provisional layers

2.2.1 Classify samples/intervals (type, index properties). ←CP

2.2.2 Behaviour-type from in-situ tests to fill gaps.

2.2.3 Provisional layer boundaries (single-location).

2.2.4 Reconcile log vs test-based classification; flag mismatches.

2.2.5 Tag special materials (fill, organic, sensitive, expansive, collapsible, cemented). ←CP

🔒 Gate 2.2.

2.3 — Derive parameters (scenario-indexed)

2.3.1 Determine scenario per parameter × layer (soil, drainage, data available).

2.3.2 Filter library to applicable methods. ←CP default. If the Code Pack is silent (no sanctioned method), surface an applicable method from the wider library (other code / research), flagged outside selected standard, for approval.

2.3.3 Compute default + alternatives; show value, scatter, reliability flag, source.

2.3.4 Co-present multi-test estimates of the same parameter.

2.3.5 Apply no-data fallbacks with low-confidence flags; surface "not recommended".

2.3.6 Assemble per-location parameter profiles.

🔒 Gate 2.3.

### Site-wide (across all locations)

2.4 — Consistency & cross-location reconciliation

2.4.1 Spatially register all locations; build layout.

2.4.2 Cross-test consistency at near/co-located soundings.

2.4.3 Lateral-variability analysis; detect outlier/anomalous locations.

2.4.4 Compare vs prior/nearby/regional/geology. ←CP priors

2.4.5 Flag conflicts; propose reconciliation with rationale.

2.4.6 Consistency summary.

🔒 Gate 2.4.

2.5 — Ground-model assembly

2.5.1 Correlate layers across locations → geotechnical units.

2.5.2 Build sections / fence diagrams / surfaces (2D/3D); interpolate; show uncertainty.

2.5.3 Groundwater regime across the site.

2.5.4 Assign each unit a representative parameter set.

2.5.5 Identify design-critical features (weak lenses, rockhead, voids, variable fill).

🔒 Gate 2.5.

2.6 — Characteristic / design value (judgment gate)

2.6.1 Assemble the value population per unit × parameter.

2.6.2 Choose the statistic per limit state & footprint (local-governing vs volume-averaged). ←CP value basis

2.6.3 Propose a cautious value + rationale, range, sensitivity.

2.6.4 Per-unit / per-parameter overrides with justification.

🔒 Gate 2.6 — engineer owns the value.

2.7 — Preliminary design recommendations

2.7.1 Design-ready profiles (fₛ–depth, q_b–depth, bearing for typical footings, subgrade reaction, earth-pressure params). ←CP correlations + safety format

2.7.2 Zone across the site with ranges.

2.7.3 Constraints, risks, further-investigation notes.

2.7.4 Compile interpretive output.

🔒 Gate 2.7.

2.8 — Package / hand-off

2.8.1 Assemble ground model + parameters + recommendations + audit trail → design stages & report.

2.8.2 Compile the Notices register at the end of the report — cautions, assumptions, low-confidence items, method choices, out-of-code methods, conflicts resolved, and recommendations (see ../03-workflow-and-human-ai-model.md → Notices register).

## Next

✓ Task-level procedures for 2.0–2.8 are now in `02-interpretation-procedures.md` — every task as In/Does/Out/Gates, with the consistency engine (2.4) and ground-model/value (2.5/2.6) given the deeper treatment.

Then apply this same per-pack treatment to a design module (shallow foundation or axial pile).

# Stage 2 — Interpretation: task procedures

The operational "how" for every task in the 2.0–2.8 breakdown of `02-interpretation.md`. That document is the architecture and reference map (the what and which clause); this one specifies, per task, the procedure the system runs.

## Per-task template

Each numbered task is specified as:

In — what it consumes (upstream task output · input-contract upload · Code Pack).

Does — the AI + engine logic, in order. The engine does deterministic arithmetic/normalization; the AI scopes, selects, explains, and drafts.

Out — what it produces and hands to the next task.

Gates/triggers — exception triggers (see `../03-workflow-and-human-ai-model.md` → Gate model) that can fire mid-task. Each step ends with its 🔒 step gate.

Legend: ←CP = supplied/parameterized by the selected Code Pack · 🔒 = step gate. Exception severities: B blocking · D decision · A advisory.

## 2.0 — Stage entry & data scoping (runs once)

### 2.0.1 — Detect entry point

In: the project session (Stage 0 code selection + Stage 1 dataset if present), or a direct upload when the engineer jumps in at Stage 2 (input contract: digitized test data).

Does:

1. Determine the entry mode — fresh from Stage 1 vs jump-in. (Per D15, flexible entry.)

2. For jump-in, read the input contract for Stage 2 and check the upload satisfies it (digitized per-location test data with depths, datums, test types).

3. Resolve the active Code Pack (from Stage 0) so all downstream ←CP hooks bind.

Out: an entry-mode flag, the bound Code Pack, and a manifest of what was supplied vs what must still be requested.

Gates/triggers: Missing/insufficient data (B) if the input contract is unmet (e.g. no datum, no test-type tags). Assumption required (D) if Stage 0 was skipped and the Code Pack must be inferred — surface the inferred pack for confirmation.

### 2.0.2 — List data needed; check presence; ingest missing; extract & map

In: the entry manifest from 2.0.1; any uploaded files (any format — PDF logs, AGS, CPT exports, lab CSVs, scans).

Does:

1. From the Code Pack + selected design modules, derive the data-needs list (which tests/parameters each downstream step will want).

2. Diff needs vs present; for each gap, choose: ask the engineer, derive later, or estimate with a flag (per D13).

3. Ingest uploads: extract structured records (test, depth, value, units, method), map vendor/lab field names to the core raw-data model, and preserve provenance (file, page/row).

Out: a structured, unit-normalized raw dataset keyed by location × depth × test, plus a gap register (ask / derive / estimate).

Gates/triggers: Error/validation failure (B) on corrupt or unparseable inputs. Missing/insufficient data (B) for any gap a downstream step hard-requires. Assumption required (D) where a field had to be inferred during mapping (e.g. assumed units).

### 2.0.3 — Register the investigation

In: the structured dataset from 2.0.2.

Does:

1. Build the investigation register: every location with coordinates, ground/collar level, test types present, depth range, and dates.

2. Normalize coordinates/levels to a single project datum (record the source datum and any transform applied).

3. Sanity-check the layout (duplicate IDs, coincident coordinates, impossible levels).

Out: the investigation register (the spatial backbone reused by 2.4–2.5) and a per-location index into the raw dataset.

Gates/triggers: Inconsistency/conflict (D) on duplicate IDs or coordinate/level clashes. Assumption required (D) if a collar level or datum had to be assumed.

🔒 Gate 2.0 — confirm dataset & scope. The engineer reviews the investigation register, the data-needs/gap list, and the bound Code Pack, then approves the dataset and scope before the per-location loop runs.

## Per-location loop — runs independently at every borehole / CPT / test

Tasks 2.1–2.3 execute once per sounding. Outputs are per-location and stay separate until the site-wide steps (2.4+) combine them.

## 2.1 — Correct & normalize

### 2.1.1 — Identify test types & depths

In: one location's raw records from 2.0.

Does: enumerate the test types present (SPT, CPT/CPTu, DMT, PMT, field vane, lab samples…), their depth coverage, sampling interval, and any depth gaps.

Out: a per-location test inventory with depth coverage per test.

Gates/triggers: Missing/insufficient data (A) if coverage is sparse relative to what later derivation needs (advisory here; may escalate at 2.3).

### 2.1.2 — Validate raw data ←CP

In: the per-location raw records + test inventory.

Does:

1. Run validity checks per test type: units, monotonic/continuous depth, physical ranges, gaps, spikes/dropouts (e.g. CPT q_c sign/zero-offset, SPT N out of range, vane torque dropouts). Range envelopes are ←CP / Method-Library metadata.

2. Flag each suspect reading with a reason and a proposed treatment (drop, clip, interpolate, keep-with-note).

Out: a cleaned raw profile with a per-reading validity flag and an exceptions list.

Gates/triggers: Error/validation failure (B) for out-of-range/corrupt readings the engineer must adjudicate. Low confidence (A) for noisy intervals carried forward.

### 2.1.3 — Apply corrections/normalizations ←CP / library

In: the cleaned raw profile.

Does: run the deterministic correction engines the Code Pack sanctions for each test, e.g.:

SPT → N₆₀ (energy/rod/borehole) and (N₁)₆₀ (overburden) — EC7 Annex F / Skempton 1986. ←CP

CPT → q_t (pore-pressure correction), F_r, and normalized Q_t/F_r — Lunne et al 1997. ←CP

DMT → I_D, K_D, E_D — Marchetti 1980. ←CP

Vane → peak/remoulded, sensitivity. ←CP

Each correction records its formula, inputs used, and source clause.

Out: corrected/normalized per-location profiles, each value clause-traceable.

Gates/triggers: Missing/insufficient data (D) when a correction input is absent (e.g. no hammer-energy ratio → AI offers a Code-Pack default to approve). Multiple routes (D) if the pack sanctions >1 normalization and the choice is consequential. Code Pack silent (D) → propose a library normalization flagged outside selected standard.

### 2.1.4 — Reconcile depth datum & water level

In: corrected profiles + the investigation register (collar levels, datum).

Does:

1. Convert all depths to the common project datum (depth-below-ground ↔ reduced level).

2. Place observed groundwater / piezometric readings (standpipe, CPTu u₂, drilling observations) on the same datum; note date/time and whether equilibrium.

3. Reconcile multiple water observations at one location into a per-location water note.

Out: datum-consistent profiles + a per-location groundwater observation.

Gates/triggers: Inconsistency/conflict (D) when water observations disagree (e.g. drilling level vs CPTu u₂). Assumption required (D) if equilibrium water level is assumed.

### 2.1.5 — Present corrected profiles

In: all of the above for the location.

Does: render the corrected/normalized profiles vs depth (raw vs corrected overlay, flags visible, datum and water level shown), with a short narrative of what was corrected and why.

Out: the review-ready corrected-profile view for this location.

Gates/triggers: none specific (feeds the step gate).

🔒 Gate 2.1. Engineer reviews corrected profiles, correction choices, flagged/edited readings, and the water observation; approves before classification.

## 2.2 — Classify & provisional layers

### 2.2.1 — Classify samples/intervals ←CP

In: corrected profiles + lab index/description data for the location.

Does: apply the pack-specific classification system to each sampled interval — EN ISO 14688/14689 (EC pack) or AS 1726 §6.1/§6.2 (AU pack) — using index properties (PSD, Atterberg, moisture) and descriptions. (Classification is not shared — see the Code-Pack split, divergence #1.)

Out: a classified interval log (material + class + basis) per sampled depth.

Gates/triggers: Multiple routes (A) at borderline classes (e.g. coarse/fine boundary, which differs by pack). Missing/insufficient data (A) where index data is absent and class rests on description only.

### 2.2.2 — Behaviour-type from in-situ tests to fill gaps

In: normalized in-situ data (CPT Q_t–F_r, DMT I_D) over unsampled depths.

Does: infer soil behaviour type between/around samples (e.g. CPT SBT via Lunne et al/Robertson) to fill log gaps; tag each as inferred (lower weight than lab class).

Out: a continuous behaviour-type profile complementing the sampled log.

Gates/triggers: Inconsistency/conflict (A→D) when behaviour-type contradicts the adjacent sample classification (escalates if it would move a layer boundary).

### 2.2.3 — Provisional layer boundaries (single-location)

In: the classified log + behaviour-type profile.

Does: segment the location into provisional layers where class/behaviour/parameters change; pick boundary depths; record the evidence for each boundary.

Out: a provisional single-location stratigraphy (layers with depth ranges + basis).

Gates/triggers: Multiple routes (D) where boundary placement is ambiguous and design-relevant.

### 2.2.4 — Reconcile log vs test-based classification

In: driller's/field log vs the test-/lab-based classification.

Does: compare the two; flag mismatches (e.g. log "clay" vs CPT sand-like behaviour); propose which governs and why.

Out: a reconciled classification with mismatch notes.

Gates/triggers: Inconsistency/conflict (D) on material mismatches the engineer must resolve.

### 2.2.5 — Tag special materials ←CP

In: the reconciled classification + index/description cues.

Does: flag fill, organic/peat, sensitive/quick, expansive, collapsible, dispersive, and cemented materials using pack-specific cues; attach the handling implication (e.g. "expansive → reactive-site checks downstream").

Out: special-material tags on the relevant layers/intervals.

Gates/triggers: Design-critical threshold (D) when a special material is flagged (forces engineer attention). Low confidence (A) where the tag rests on weak evidence.

🔒 Gate 2.2. Engineer reviews the provisional stratigraphy, classification basis, reconciled mismatches, and special-material tags; approves before parameter derivation.

## 2.3 — Derive parameters (scenario-indexed)

The core of the Method Library at work. Derivation = a function of { parameter × soil type × drainage × data available } (D24), evaluated per layer.

### 2.3.1 — Determine scenario per parameter × layer

In: the approved provisional stratigraphy + corrected test profiles + the parameter needs list (from 2.0.2, driven by the design modules).

Does: for each (layer × required parameter), assemble the scenario key — soil type, drainage condition (drained/undrained), and which test data actually exist at that depth.

Out: a scenario key per (layer × parameter).

Gates/triggers: Assumption required (D) when drainage condition must be assumed.

### 2.3.2 — Filter library to applicable methods ←CP

In: the scenario keys.

Does:

1. Query the Method Library for methods whose applicability metadata matches the scenario.

2. Set the Code-Pack default method; keep the rest as reachable alternatives.

3. If the Code Pack is silent (no sanctioned method for the scenario), surface an applicable method from the wider library (other code / peer-reviewed research), flagged outside selected standard (from [reference]), for approval. (D26.)

Out: a per-scenario candidate-method set with one default marked.

Gates/triggers: Multiple routes (D) when ≥2 consequential methods apply. Code Pack silent (D, out-of-code) per above. Out-of-applicability (B) if the only candidate sits outside its valid conditions.

### 2.3.3 — Compute default + alternatives; show value, scatter, reliability, source

In: the candidate-method sets + corrected data.

Does: run the derivation engines for the default and alternatives at each depth; produce value(s) vs depth, inter-method scatter, a reliability flag, and the source clause per method.

Out: per-parameter derived profiles with scatter band + reliability + provenance.

Gates/triggers: Methods disagree materially (D) when scatter exceeds threshold. Low confidence (A→D) when a reliability flag affects a governing value.

### 2.3.4 — Co-present multi-test estimates of the same parameter

In: derived estimates of the same parameter from different tests (e.g. s_u from CPTu, vane, lab).

Does: overlay the independent estimates on one depth axis; quantify agreement; propose a provisional preferred estimate (or a reconciliation note) per depth.

Out: a consolidated per-parameter view across tests, per location.

Gates/triggers: Inconsistency/conflict (D) when independent tests disagree beyond threshold.

### 2.3.5 — Apply no-data fallbacks; surface "not recommended"

In: scenarios where the preferred data is absent (e.g. clay s_u, no lab).

Does: apply the library's fallback chain (e.g. s_u ≈ f₁·N₆₀ via Stroud) with an explicit low-reliability flag; and where a route is encoded as not recommended for the scenario (e.g. clay φ′ from SPT), surface that rather than silently using it.

Out: fallback-derived values, each flagged; explicit "not recommended" notices where applicable.

Gates/triggers: Low confidence (D) — every no-data fallback is logged for the notices register. Out-of-applicability (B) if a not-recommended route would otherwise be used.

### 2.3.6 — Assemble per-location parameter profiles

In: all derived/consolidated parameters for the location.

Does: compile the per-location parameter profiles (parameter × depth, with chosen method, scatter, reliability, provenance) ready for site-wide combination.

Out: the location's complete derived-parameter profile set.

Gates/triggers: none specific (feeds the step gate).

🔒 Gate 2.3. Engineer reviews derived parameters, picks/confirms the correlation per parameter (a must-stay-human choice), accepts fallbacks/flags, and approves the per-location profiles. (This closes the per-location loop; 2.4+ go site-wide.)

## Site-wide steps — combine all locations

Tasks 2.4+ take the per-location profiles from 2.3 and reason across the whole site.

## 2.4 — Consistency & cross-location reconciliation (deep treatment)

The consistency engine is core IP (D17) and the least code-defined step — codes require the activity (EN 1997-2 §6.3 "evaluation of geotechnical information"; AS 1726 §5.1.1 iterative + §5.6.2(d) regional & local geology required) but prescribe little method. So the logic is agnostic core; only the priors framework and the obligation to consider regional/local geology are ←CP.

The central problem it must get right: a disagreement between two numbers is not automatically an error. It is one of four things, and the correct response differs for each — so the engine's job is to classify before it reconciles:

| Conflict nature | What it means | Correct response |
| --- | --- | --- |
| Measurement / process error | datum, units, equipment, transcription | correct or exclude; re-run upstream |
| Method-reliability gap | a low-reliability route disagrees with a strong one | prefer the higher-reliability derivation; down-weight the fallback |
| Genuine ground variability | the site really does change laterally/vertically | do not average it away — preserve it as a zoning driver (2.5) and possible local-governing value (2.6) |
| Prior mismatch | new data departs from regional/prior expectation | weigh quality & recency; site-specific data usually governs, but flag the departure |

Status note (open item (c)): the specific statistical machinery below — robust estimators (median/MAD), distance-weighting, optional variogram/Bayesian updating against priors — is Proposed, pending the founder's preferred references. Thresholds are engineer-configurable; default conservative (surface more rather than less, per D25).

### 2.4.1 — Spatially register all locations; build layout

In: the investigation register (2.0.3) + per-location parameter profiles (2.3.6).

Does:

1. Place every location in plan (and on candidate section lines); compute inter-location distances.

2. Group locations into comparison neighbourhoods — co-located/near sets (within a configurable radius) and broader clusters — defining who is compared with whom.

3. Establish the spatial framework reused by 2.4.2–2.4.3 and by zoning in 2.5.

Out: a spatial layout with neighbour/cluster relationships and candidate section lines.

Gates/triggers: Assumption required (D) where coordinates/levels were estimated. Inconsistency/conflict (D) for a location that plots off-site or implausibly far from its ID.

### 2.4.2 — Cross-test consistency at near/co-located soundings

In: the layout + per-location profiles, with each derivation's reliability rank (from the Method Library, set in 2.3).

Does:

1. Where ≥2 different test types are co-located/near, align their independent estimates of the same parameter onto common depth bins / provisional layers (from 2.2).

2. Compute the discrepancy per bin and classify agreement against a tolerance band that is widened by the lower reliability rank of the pair (a fallback disagreeing with a lab value is expected, not a conflict).

3. Emit a cross-test agreement view per parameter; mark genuine disagreements for 2.4.5.

Out: a cross-test agreement matrix (parameter × neighbourhood) with flagged disagreements.

Gates/triggers: Methods disagree materially (D) beyond the reliability-adjusted band. Low confidence (A) where only low-reliability routes are available to cross-check.

### 2.4.3 — Lateral-variability analysis; detect outlier/anomalous locations

In: the layout + per-location profiles.

Does:

1. For each layer × parameter, characterize spatial behaviour — trends/gradients (deepening, thickening), and the natural spread — using robust statistics (median/MAD) so a single bad point doesn't distort the picture.

2. Detect locations that deviate from their neighbours beyond expected variability, and classify each candidate: likely error, genuine local feature (weak lens, fill pocket, rockhead high), or undetermined.

3. Distinguish a smooth lateral trend (carry into zoning) from a discrete anomaly (investigate or isolate).

Out: per-layer lateral-variability characterization + an outlier/anomaly list with a provisional cause for each.

Gates/triggers: Inconsistency/conflict (D) for outliers needing adjudication. Design-critical threshold (D) when an anomaly is a weak feature that could govern a limit state (forces review even if it is "just one point").

### 2.4.4 — Compare vs prior / nearby / regional / geology ←CP priors

In: per-location profiles + the priors framework (←CP): regional geology, published local correlations, prior SI on the site, and any adjacent-project data the engineer supplies.

Does:

1. Compare the new stratigraphy and parameter ranges against the expected ranges from regional geology / prior data.

2. Flag departures from regional expectation (e.g. strengths well outside the published band for the formation).

3. ←CP obligation: EC pack frames this as EN 1997-2 §6.3 evaluation; AU pack enforces AS 1726 §5.6.2(d) (regional & local geology required in the factual report).

Out: prior-comparison notes; departures flagged for 2.4.5.

Gates/triggers: Inconsistency/conflict (D) on departures from priors. Low confidence (A) where priors are weak/absent. Assumption required (D) if a regional model had to be assumed.

### 2.4.5 — Flag conflicts; propose reconciliation with rationale (the core)

In: every flagged disagreement from 2.4.2–2.4.4.

Does: for each conflict —

1. Classify its nature against the four-way table above.

2. Apply the reconciliation ladder (below) to produce a proposed resolution.

3. Attach rationale + confidence + impact (does it touch a governing parameter/zone?).

Reconciliation ladder (agnostic core):

1. Error? → identify source (datum, units, equipment, transcription); correct or exclude; re-run the affected upstream task. Log it.

2. Method-reliability gap? → prefer the higher-reliability derivation per the library ranking; down-weight/retire the fallback; keep both visible.

3. Genuine ground variability? → preserve it — do not average across it. Tag it as a zoning driver for 2.5 and, if it governs locally, a candidate local value for 2.6.

4. Prior mismatch? → weigh data quality & recency; site-specific data normally governs the regional prior, but record the departure (and revisit the prior if many points depart).

5. Undetermined or design-critical? → recommend further investigation; meanwhile carry the conservative interpretation forward with an explicit flag.

Out: a conflict register — each conflict with nature, proposed reconciliation, rationale, confidence, impact.

Gates/triggers: Inconsistency/conflict (D) — the engineer adjudicates each (this is the must-stay-human resolution called out in 02-interpretation.md → Human gates (b)). Design-critical threshold (D). Methods disagree (D).

### 2.4.6 — Consistency summary

In: the agreement matrix, anomaly list, prior-comparison notes, and conflict register.

Does: compile a site-wide consistency summary — what agrees, what conflicted and how it is proposed to reconcile, residual uncertainty, and any further-investigation recommendations; stage the corresponding entries for the Notices register (D27).

Out: the consistency summary section + staged notices entries.

Gates/triggers: none specific (feeds the step gate).

🔒 Gate 2.4. Engineer reviews the conflict register, decides each reconciliation, and approves a site-wide-consistent dataset before ground-model assembly. Decisions and any carried-forward flags pass to the notices register.

## 2.5 — Ground-model assembly (deep treatment)

The ground model is the hub of the whole spine — everything funnels into it and both design modules read from it (../03 spine). 2.5 converts the reconciled, per-location data from 2.4 into one spatial site model: **geotechnical units + geometry + groundwater + per-unit parameters

flagged critical features.* Agnostic core = the assembly logic and the unit* concept; ←CP =

the classification system feeding unit definition and the deliverable framing (EC: EN 1997-1 §3 / EN 1997-2 §6; AU: AS 1726 §5.2 geotechnical model, §5.6 factual report).

Key concept — the geotechnical unit. A unit is an engineering grouping (similar behaviour + parameters + stratigraphic position), which may merge or split the provisional single-location layers from 2.2. The genuine variability preserved by 2.4 constrains this: never merge a weak lens into a strong unit just because it is thin.

### 2.5.1 — Correlate layers across locations → geotechnical units

In: reconciled per-location stratigraphy + parameters (post-2.4), the spatial layout (2.4.1), and the conflict register's variability tags (2.4.5).

Does:

1. Match provisional layers across locations by classification + behaviour-type + parameter signature + stratigraphic position.

2. Group correlated layers into geotechnical units — merge where consistent, split where 2.4 flagged genuine variability or a weak feature.

3. Record each unit's defining criteria and the locations evidencing it.

Out: a site-wide set of geotechnical units (membership + defining criteria).

Gates/triggers: Multiple routes (D) where cross-location correlation is ambiguous (is the sand at BH1 the same body as at BH3?). Inconsistency/conflict (D) where layers won't correlate cleanly. Design-critical threshold (D) when defining a weak/governing unit.

### 2.5.2 — Build sections / fence diagrams / surfaces (2D/3D); interpolate; show uncertainty

In: geotechnical units + spatial layout + candidate section lines.

Does:

1. Build cross-sections / fence diagrams along section lines; interpolate unit boundaries between locations.

2. Optionally build boundary surfaces (top-of-unit, rockhead) by interpolation across the site (2D/3D).

3. Show interpolation uncertainty — wide bands where data is sparse — and flag extrapolation beyond the data envelope.

Out: cross-sections + boundary surfaces with uncertainty bands and extrapolation flags.

Gates/triggers: Low confidence (A→D) in sparsely-constrained zones that carry design. Assumption required (D) where boundaries are interpolated across large gaps. Out-of- applicability (A) flag on extrapolation beyond the data.

### 2.5.3 — Groundwater regime across the site

In: per-location water observations (2.1.4), CPTu u₂/dissipation, standpipe/piezometer data, and regional hydrogeology (←CP priors).

Does:

1. Combine the per-location observations into site groundwater surface(s); distinguish phreatic vs piezometric / perched / artesian, and equilibrium vs short-term.

2. Establish the design groundwater regime, including seasonal/long-term high where available; flag assumptions where data is thin.

3. Reconcile disagreeing water observations (reusing the 2.4 reconciliation logic).

Out: a site groundwater model (surface(s) + regime + design-level assumptions).

Gates/triggers: Inconsistency/conflict (D) on disagreeing observations. Assumption required (D) for the design water level. Design-critical threshold (D) — water level governs effective stress / buoyancy, so it always forces review.

### 2.5.4 — Assign each unit a representative parameter set

In: per-location derived parameters (2.3) grouped by unit; the unit definitions.

Does:

1. Pool each unit's member-location values into a unit population per parameter (the bridge to 2.6).

2. Assign a provisional representative profile/value per unit — constant or depth-trend. This is descriptive (central tendency + spread), not the cautious characteristic value (that judgment is 2.6).

3. Support multiple constitutive models per unit (model-agnostic, D16) — the parameter set depends on which model(s) the downstream modules consume.

Out: each unit carrying a representative parameter population + provisional central profile, per constitutive model.

Gates/triggers: Methods disagree (A) carried from 2.3. Low confidence (A) for thin populations.

### 2.5.5 — Identify design-critical features

In: units + sections + variability/anomaly tags (2.4) + special-material tags (2.2.5).

Does: explicitly call out features that can govern design — weak lenses / soft bands, rockhead level & variability, voids / karst / old workings, variable or uncontrolled fill, perched water, expansive / collapsible / reactive zones — each with its design implication and where it bites.

Out: a design-critical features register attached to the ground model.

Gates/triggers: Design-critical threshold (D) per feature. Out-of-applicability (B) if a feature breaks a downstream method's assumptions (e.g. karst under a spread-footing method).

🔒 Gate 2.5. Engineer reviews units, sections/surfaces + uncertainty, the groundwater model, representative parameters, and critical features; approves the ground model — the master input feeding both design modules and the report.

## 2.6 — Characteristic / design value (judgment gate, deep treatment)

The value basis is the deepest Code-Pack split (divergence #2) and one of the three must-stay-human gates: EC7 §2.4.5.2's "cautious estimate" is judgment, not a formula. Agnostic core = (i) assemble the population per unit × parameter, (ii) the engineer owns the final value. ←CP = the value basis itself — what kind of value design wants, and how safety is then applied:

|  | EC pack | AU pack |
| --- | --- | --- |
| Value design wants | characteristic value Xₖ = cautious estimate of the value affecting the limit state (EN 1997-1 §2.4.5.2); statistical aid Schneider 1997 ◇ / national annex | cautiously-assessed (ultimate) geotechnical strength (no single "characteristic" clause) |
| Safety applied later | partial factors per DA1/DA2/DA3 (Stages 4–6) | risk-based φg strength-reduction factor → design geotechnical strength (AS 2159 §1.3.7 piles · AS 5100.3 §7.3.5 footings) |

So the cautious value lands at a different point in the chain and is factored differently — but the upstream population-assembly and the human-ownership rule are identical. Steps 2.6.1–2.6.4 are agnostic; only the meaning of "cautious value" and the downstream safety hook are ←CP.

### 2.6.1 — Assemble the value population per unit × parameter

In: the unit parameter populations from 2.5.4 + reliability/provenance metadata.

Does:

1. Assemble the full population of derived values per unit × parameter (across locations & tests), each tagged with reliability, test type, depth, location.

2. Compute robust descriptive statistics (n, central tendency, spread, depth-trend), weighted by reliability; display the population and residual scatter.

3. Flag thin populations (low n) where statistics are unreliable.

Out: a per-unit × parameter value population + descriptive statistics.

Gates/triggers: Low confidence (A→D) for low-n populations affecting a governing value.

### 2.6.2 — Choose the statistic per limit state & footprint ←CP value basis

In: the populations + the design modules' limit states + footprint (or an assumption).

Does:

1. Determine whether the governing limit state is sensitive to a local minimum (pile tip in a thin bearing layer, weak lens under a strip) or to a spatial average over a large volume (raft / embankment averaging out variability).

2. Select the statistic accordingly — local-governing → cautious estimate of the low value (low fractile); volume-averaging → cautious estimate of the mean (lower uncertainty, because averaging reduces variance). ←CP defines exactly what "cautious estimate" / "cautiously assessed strength" means in the active pack.

3. Where footprint is unknown at interpretation time, default to the local (more conservative) basis and flag the assumption.

Out: the chosen statistic + spatial basis per unit × parameter × use.

Gates/triggers: Assumption required (D) when footprint/limit state must be assumed. Multiple routes (D) where both bases are plausible. Design-critical threshold (D).

### 2.6.3 — Propose a cautious value + rationale, range, sensitivity

In: the population + chosen statistic + spatial basis.

Does:

1. Compute the proposed cautious value (EC: characteristic Xₖ via §2.4.5.2, statistical aid optional; AU: cautiously-assessed strength).

2. Give rationale (how cautious and why), the plausible range (e.g. mean → low fractile), and a sensitivity note (how far the design result moves if the value shifts within range).

3. Show provenance: which population, which statistic, which clause.

Out: a proposed cautious value per unit × parameter, with rationale + range + sensitivity.

Gates/triggers: Design-critical threshold (D) — this value governs. Low confidence (A→D).

### 2.6.4 — Per-unit / per-parameter overrides with justification

In: the engineer's edits.

Does: let the engineer override any value, recording the justification (experience, regional knowledge, deliberate conservatism, extra data); recompute downstream sensitivity; log the override.

Out: the finalized characteristic / design-basis value set, each with provenance or override justification.

Gates/triggers: Assumption required (D) to capture each override's justification.

🔒 Gate 2.6 — engineer owns the value. A must-stay-human judgment gate (§2.4.5.2 cautious estimate is judgment, not a formula). The engineer reviews each proposed value, basis, range, and sensitivity, edits/approves, and the approved set becomes the design-basis parameter set carried to Stages 4–6 and the report. Every choice feeds the notices register.

## 2.7 — Preliminary design recommendations

The bridge interpretation → design (D18), Geotechnical-Interpretive-Report style. It produces geometry-independent, design-ready profiles from the ground model + design-basis values, using the pack's sanctioned direct field-test correlations and safety format — explicitly flagged preliminary. ←CP = which correlations are sanctioned + the safety format applied.

Guardrail — division of labour. 2.7 gives capacity/parameter profiles per unit/depth, independent of a specific foundation geometry or load. The geometry-and-action-specific check is the design stage (Stage 5/6). The output is labelled indicative; it must not be mistaken for the final design.

### 2.7.1 — Design-ready profiles ←CP correlations + safety format

In: the design-basis parameter set (2.6) + ground model (2.5) + groundwater (2.5.3) + the pack's design correlations and safety format.

Does:

1. Compute geometry-independent design profiles per unit/depth:

unit skin friction fₛ vs depth (pile shaft) and base resistance q_b vs depth;

bearing for typical shallow-footing types/widths;

subgrade reaction modulus; earth-pressure parameters (K₀/Kₐ/K_p). (MVP modules consume fₛ/q_b/bearing first; the rest round out the GIR output.)

2. Use ←CP sanctioned correlations — EC: EN 1997-2 §4.3.4/§4.4.4/§4.6.4/§4.10.4/§4.11 direct field-test design, piles NEN 6743 / Fascicule 62, settlement Schmertmann 1970 / Burland & Burbidge 1985; AU: fₛ/q_b per AS 2159, bearing per AS 5100.3.

3. Apply the pack's safety format at the preliminary level, clearly labelling what is characteristic/ultimate vs factored (EC: characteristic + indicative partial-factored; AU: φg-reduced design geotechnical strength).

Out: geometry-independent design-ready profiles per unit, each with method + clause + safety-format basis + preliminary flag.

Gates/triggers: Multiple routes (D) where ≥2 correlations apply. Methods disagree materially (D) — expected for CPT pile methods (scatter is the norm). Code Pack silent (D, out-of-code). Design-critical threshold (D).

### 2.7.2 — Zone across the site with ranges

In: the profiles + the site zoning (units/sections from 2.5).

Does: present the profiles zoned spatially (per geotechnical zone/section), each as a range reflecting variability and uncertainty (not a single deterministic line), consistent with the local-governing vs volume-averaged basis chosen in 2.6.

Out: zoned design-ready profiles with ranges.

Gates/triggers: Low confidence (A) in poorly-constrained zones. Design-critical threshold (D) where a zone governs.

### 2.7.3 — Constraints, risks, further-investigation notes

In: design-critical features (2.5.5), consistency residuals (2.4.6), thin-population / low-confidence flags, special-material tags (2.2.5).

Does: compile geotechnical constraints and risks — settlement-sensitive units, negative skin friction potential, aggressive ground/chemistry, construction issues (base heave, running sand), liquefaction/seismic susceptibility flags — plus explicit further-investigation recommendations where data is insufficient.

Out: a constraints / risks / recommendations list.

Gates/triggers: Design-critical threshold (D). Low confidence → further-investigation recommendation (logged for the notices register).

### 2.7.4 — Compile interpretive output

In: all of 2.7.

Does: assemble the GIR-style interpretive output — narrative + zoned design profiles + constraints/risks — marked preliminary/indicative and pointing to the design stages for the geometry-and-action-specific check.

Out: the preliminary-design-recommendations package.

Gates/triggers: none specific (feeds the step gate).

🔒 Gate 2.7. Engineer reviews the design-ready profiles, zoning, correlation choices, safety-format basis, and constraints/risks; approves the preliminary design recommendations. (This is the Stage-2 sign-off in the spine: "approved derived parameters + recommendations".)

## 2.8 — Package / hand-off

Closes interpretation. Two mechanical jobs over already-approved material — so no new step gate: everything in 2.8 was signed off at Gates 2.5–2.7; the package is then consumed by the design stages and the final report sign-off (Stage 7).

### 2.8.1 — Assemble package → design stages & report

In: the ground model (2.5), the design-basis value set (2.6), the recommendations (2.7), and the full audit trail (every gate approval, method choice, override, assumption — P5).

Does:

1. Package the deliverables to satisfy the input contracts of the downstream design stages (Stage 5 shallow foundation ← characteristic params + bearing profiles; Stage 6 axial pile ← fₛ/q_b profiles) — per D15, the same package works whether design runs in-app or is exported.

2. Bundle the audit trail: method + inputs + assumptions + citations + sign-off per item.

3. Version/freeze the ground model as the master input; later upstream changes trigger downstream re-validation.

Out: the interpretation hand-off package (ground model + design-basis params + recommendations + audit trail), contract-shaped for design + report.

Gates/triggers: Missing/insufficient data (B) if a downstream input contract is unmet (a parameter a design module requires was never derived) → loop back to 2.3/2.6.

### 2.8.2 — Compile the Notices register

In: every notice staged across 2.0–2.7.

Does: compile the structured Notices register (D27, ../03 → Notices register) — each entry: item · location/context · trigger · options considered · decision · rationale · approver · date — grouped for the end of the report: assumptions, low-confidence / no-lab fallbacks, method choices where ≥2 routes existed, conflicts resolved, missing-data estimates, out-of- applicability overrides, out-of-code methods, design-critical items, and further-investigation recommendations. Flag any still-open items.

Out: the Notices register section for the report.

Gates/triggers: a final completeness check surfaces any unresolved blocking item (normally none — gates resolve inline).

## Status — complete

Installment 1 — per-location loop (2.0–2.3) ✓

Installment 2 — 2.4 consistency engine (deep) ✓

Installment 3 — 2.5 ground model + 2.6 characteristic value (deep) ✓

Installment 4 — 2.7 preliminary design recommendations + 2.8 package / notices ✓

All of 2.0–2.8 now have task-level procedures. Next planned work (per ../06-decision-log.md): apply this same per-pack, In/Does/Out/Gates treatment to a design module (shallow foundation or axial pile).